GeM | Government eMarketplace

Bid Document

# 1. Bid Details

| Bid Number | GEM/2026/B/9852104 |
| --- | --- |
| Bid End Date/Time | 24-08-2026 15:00:00 |
| Bid Opening Date/Time | 24-08-2026 15:30:00 |
| Bid Offer Validity | 180 Days |
| Ministry/State Name | Ministry of Petroleum and Natural Gas |
| Department Name | Refineries & Petrochemicals Division |
| Organisation Name | Indian Oil Corporation Limited (IOCL) |
| Office Name | Gujarat Refinery, Vadodara |
| Item Category | Controlled Bolting Equipment & Turnaround Services |
| Total Quantity | 1 Package (Comprehensive Service & Supply) |
| Contract Period | 3 Year(s) |
| MSE Exemption for Years of Experience | Yes |
| Startup Exemption for Years of Experience | Yes |
| Document Required from Seller | Experience Criteria, Past Performance, OEM Authorization, Compliance Certificate |
| Bid to RA Enabled | Yes |
| RCM Applicable | No |
| EMD Detail | Applicable (2% of estimated value) |
| ePBG Detail | Applicable (5% of contract value) |

# 2. EMD & ePBG Details

Earnest Money Deposit (EMD) of 2% of the total estimated contract value must be submitted in the form of Bank Guarantee / Demand Draft favoring the Accounts Officer, OPGC Banharpali. Bidders seeking exemption under MSE / Startup categories must upload valid Registration Certificates issued by NSIC / UDYAM / DPIIT.

# 3. Splitting & Purchase Preference

- Splitting of Order: The Buyer reserves the right to split the order among multiple qualified L1, L2 sellers in the ratio of 60:40, subject to L2 matching L1 prices.

- MII Purchase Preference: Preference will be given to Class 1 Local Suppliers as defined under the Make in India policy. Local content minimum requirement is 50%.

- MSE Purchase Preference: 25% of total procurement quantity is reserved for Micro and Small Enterprises.

# 4. Vendor Eligibility & Qualification Criteria

The bidder or its OEM must have executed at least 3 similar contracts in major public sector undertakings (PSUs), refineries, power plants, or fertilizer complexes during the last 5 financial years. Minimum annual financial turnover during the last 3 financial years must not be less than INR 2.5 Crores. ISO 9001, ISO 14001, and ISO 45001 certifications are mandatory.

# 5. Comprehensive Scope of Work (SOW)

## 5.1 Plant Shutdown & Turnaround Maintenance Bolting Services

The contractor shall mobilize certified personnel and high-capacity equipment for annual plant shutdown maintenance across Hydrocracker and Crude Distillation Units. Services demand 24/7 technical execution for controlled bolting on high-pressure reactor vessel heads, heat exchanger end covers, and steam reformer piping.

## 5.2 Supply & Operation of Hydraulic Torque Wrench Systems

Contractor must deploy low-profile and square-drive hydraulic torque wrench tools capable of working up to 35,000 Nm torque output. All hydraulic torque wrenches must feature multi-axis swivel couplings, anti-reverse ratchets, and digital electric power packs to ensure exact torque application during flange joint tightening.

## 5.3 Hydraulic Bolt Tensioning & Pre-tensioning Protocol

For critical high-temperature flanges, the vendor must supply multi-stage hydraulic bolt tensioning equipment and specialized bolt tensioner units. Execution of stud bolt tensioning must adhere strictly to ASME PCC-1 Guidelines for Bolted Flange Joint Assembly to achieve uniform pre-tensioning and prevent joint leakage.

## 5.4 Flange Joint Integrity & Flange Management Program

The scope encompasses a total flange management service including joint tagging, ultrasonic bolt elongation measurement, flange face inspection, gasket replacement, nut splitter deployment for corroded fasteners, and final flange joint integrity testing prior to plant pressurization.

# 6. Technical Specifications & BOQ

The equipment and tools supplied under this contract must conform strictly to the technical parameters specified below. Any deviation will result in immediate rejection during pre-dispatch inspection.

| Item # | Description of Tool / Service | Technical Standard / Parameter | Qty |
| --- | --- | --- | --- |
| 1 | Hydraulic Torque Wrench (Square Drive) | Max Torque 15,000 Nm, 1.5 inch drive | 4 |
| 2 | Low Profile Hydraulic Torque Wrench | Hex sizes 50mm to 110mm | 6 |
| 3 | Top-Side Hydraulic Bolt Tensioner | Max Working Pressure 1500 Bar | 8 |
| 4 | Hydraulic Nut Splitter Unit | Cuts M24 to M64 heavy hex nuts | 2 |
| 5 | High-Pressure Electric Torque Pump | 700 Bar output, 230V, quad port | 3 |
| 6 | Flange Joint Integrity Test Kit | Nitrogen/Helium leak testing skid | 1 |

# 7. Buyer Added Bid Specific Terms & Conditions

- Inspection & Testing: Pre-dispatch inspection (PDI) shall be conducted by the Buyer's authorized third-party inspection agency (TUV / DNV / Lloyd's Register) at OEM works.

- Calibration Certificates: All calibration must be conducted using NABL accredited master instruments with valid calibration certificates valid for 12 months from the delivery date.

- On-Site Support & Training: Successful bidder must depute qualified service engineers for on-site commissioning, tool operation training, and maintenance demonstration for a minimum of 5 working days.

- Guarantee / Warranty: The seller shall provide a comprehensive 36-month replacement warranty against manufacturing defects, material failure, or performance degradation under full rated load.

- Liquidated Damages (LD): If the seller fails to deliver tools or services within the specified schedule, LD at 0.5% per week up to a maximum of 10% of total contract value will be deducted.

- Safety & Statutory Compliance: All field technicians deputed for turnaround services or plant shutdown work must possess valid Plant Safety Cards, PPE kits, and medical fitness certificates.

- Force Majeure: Standard GeM SLA terms regarding natural disasters, strikes, or government restrictions shall govern contract extensions.

- Payment Terms: 80% payment upon receipt and physical verification of tools at site; remaining 20% against submission of Performance Bank Guarantee (ePBG).

# Annexure I — Technical Compliance & Self-Declaration Undertaking

To,

The Superintending Engineer (Procurement & Contracts),

Indian Oil Corporation Limited (IOCL),

Gujarat Refinery, Vadodara

Subject: Technical Compliance & Undertaking for GeM Bid No. GEM/2026/B/9852104

Dear Sir,

We hereby declare that we have carefully read and understood all technical specifications, scope of work, and buyer terms contained in the GeM Bid document. We confirm full compliance without any commercial or technical deviations.

We confirm that all equipment supplied (including hydraulic tools, tensioners, multipliers, and joint integrity testing units) will be 100% genuine, brand new, and supported by OEM test certificates.

We further declare that our firm has not been blacklisted or debarred by any Central / State Government department or PSU as on the bid submission date.

Thanking you,

Yours faithfully,

For & On Behalf of: ______________________________

Authorized Signatory: ______________________________

Name & Designation: ______________________________

Company Seal & Date: ______________________________

GeM Bid Reference: GEM/2026/B/9852104  |  Generated via GeM Portal  |  Confidential